4RUNNER TELEMETRY BUILD

A/C Pressure • BLE/Wi-Fi • SD Logging • Agentic PCB

Master Hardware + Software PRD — v1.0 — August 16, 2026

MISSION

Build a permanent engine-bay telemetry platform for the 2005 Toyota 4Runner beginning with A/C high/low pressure and temperature. The node logs independently, talks to an iPhone over BLE, exposes a Wi-Fi access point, self-tests, supports safe OTA updates, and can be manufactured as a custom JLCPCB PCBA.

ARCHITECTURE

V0: Arduino-programmable ESP32-S3 prototype in an IP-rated enclosure.

V1: custom carrier PCB with protected 12 V power, pressure/temperature inputs, microSD, sealed connectors and self-test.

V2: smaller integrated node and optional coin-cell BLE pressure pucks.

Vehicle 12 V -> fuse -> reverse/transient protection -> buck regulator -> ESP32-S3 -> pressure + temperature -> microSD. ESP32-S3 supplies BLE, Wi-Fi AP, web dashboard and OTA.

COMPLETE PROTOTYPE HARDWARE

• ESP32-S3 DevKitC or compact ESP32-S3 board.

• R-134a high/low quick couplers, one pair.

• Two R-134a/PAG-compatible pressure transducers.

• Two or three sealed temperature probes.

• microSD socket/module plus high-endurance 8–32 GB card.

• IP65/IP67 gasketed polycarbonate/ABS enclosure.

• Deutsch DT/DTM or equivalent sealed connector kits.

• 18 AWG TXL automotive wire for power/ground; 20–22 AWG TXL/GXL for sensors.

• High-temp split loom/braided sleeve and adhesive-lined heat shrink.

• Inline fuse holder plus 1–3 A fuse and correct ring terminals.

• M3 stainless PCB screws/standoffs; M4/M5 stainless bracket bolts with nyloc nuts.

• Rubber vibration isolators/grommets and IP67 cable glands/bulkhead connectors.

• Automotive-suitable buck regulator, TVS transient suppression and reverse-polarity protection.

• USB-C recovery connection, aluminum/stainless bracket, refrigerant-rated adapters/seals and harness labels.

A/C MECHANICAL INTERFACE

Prototype chain: vehicle R-134a service port -> correct HIGH/LOW quick coupler -> rated adapter -> pressure transducer. Keep high/low fittings keyed and labeled. Typical service couplers use 1/4-inch SAE flare. Minimize mass hanging from aluminum A/C lines; mechanically support heavy sensor stacks or use a deliberately selected rated short remote hose. Leak-check every new joint. Long-term road use should graduate from inexpensive service-tool couplers to a qualified permanent fitting strategy. Confirm every wetted material and seal against R-134a and PAG oil.

ENGINE-BAY MOUNTING / HARNESS

Body/chassis mount the electronics enclosure. Avoid exhaust/manifold heat, belts, fans, radiator hot-air discharge and direct spray. PCB mounts on M3 standoffs. Use rubber vibration isolation where practical, stainless mounting hardware, factory-style harness routing, service loops, sealed connectors, TXL/GXL wire, abrasion loom and strain relief. Fuse close to the 12 V source and prefer ignition-switched power unless parked logging is required.

V1 PCB REQUIREMENTS

12 V protected input with reverse polarity, TVS/transient suppression and regulated rails; ESP32-S3 with antenna keep-out and USB/UART recovery; two protected pressure interfaces; 2–4 temperature inputs; on-board microSD; LED, service button and labeled test pads; optional CAN/OBD-II expansion. Prefer JLCPCB/LCSC-available parts and generate BOM, CPL, Gerbers, STEP, net list and machine-readable firmware pin map.

PCB SELF-TEST

POST sequence: reset reason and revisions -> VIN/rail ADC measurements -> MCU/brownout health -> sensor enumeration -> analog open/short/plausibility checks -> SD mount/write/readback/checksum -> BLE start -> Wi-Fi AP start -> persist report. Continuous BIT checks pressure range/rate/stuck values, high/low relationship plausibility, temperature plausibility, SD latency/errors/free space, rail min/max, brownouts/resets, BLE/Wi-Fi counters and per-task watchdog heartbeats. Factory mode emits JSON such as {"board_rev":"A1","fw":"0.1.0","sd":"PASS","pressure_high":"PASS","pressure_low":"PASS","ble":"PASS","wifi":"PASS","overall":"PASS"}.

FIRMWARE

SensorTask acquires/calibrates/timestamps. HealthTask runs POST/BIT/watchdog. LogTask handles buffered SD writes/rotation/recovery. BLETask handles advertisements and GATT. WiFiTask provides AP + HTTP/WebSocket dashboard. OTATask manages authenticated update/version/rollback. ConfigTask persists calibration/sample rates/device profile. TimeTask synchronizes time. CommandTask provides factory/service commands.

SD DATA

CSV fields: timestamp_ms, pressure_high_psi, pressure_low_psi, temp_high_c, temp_low_c, temp_ambient_c, vin_v, v3v3_v, compressor_state, sd_status, ble_clients, wifi_clients, fault_mask, sample_counter.

Files: /config/device.json; /logs/YYYY-MM-DD.csv; /events/YYYY-MM-DD.jsonl; /diagnostics/latest_post.json; /diagnostics/health_history.jsonl; /firmware/current_manifest.json.

IOS / BLE / WI-FI

BLE advertisements carry latest pressure, temperatures, VIN, health and sample counter without a persistent connection. GATT provides high-rate graphing, calibration/config and diagnostics. iOS screens: live HIGH/LOW PSI dashboard, graphs, faults, configuration, device revision, sessions/logs, CSV/JSON export and firmware/service. Wi-Fi AP SSID can be 4RUNNER-TELEMETRY-XXXX with browser dashboard, WebSocket stream, /api/status JSON, SD download, configuration and OTA service.

SAFE OTA

Build against pinned PCB revision/pin map; run tests; generate binary + SHA-256 + signed/versioned manifest; install inactive partition; boot probation image; run POST and timed health test; mark valid only on pass; otherwise roll back; log outcome to SD.

AGENTIC PCB -> JLCPCB

Flux is the recommended AI PCB environment. Flux documents an AI hardware agent plus MCP connectivity for external AI clients. Workflow: maintain hardware_requirements.yaml -> Flux creates/updates schematic/BOM/nets/PCB -> AI review of datasheets, power, antenna, connectors, DFM and self-test -> ERC/DRC -> Gerber/BOM/CPL/STEP/pin-map export -> independent manufacturing package check -> human purchase approval -> JLCPCB PCBA -> factory-test firmware -> automated fixture PASS/FAIL JSON -> production firmware -> enclosure/harness -> vehicle baseline. JLCPCB PCBA requires Gerber, BOM and CPL. JLCPCB's 2026 API offering can automate file upload, pricing/order workflows and tracking for approved API users.

FLUX AGENT BRIEF

Design a compact ESP32-S3 automotive telemetry carrier PCB for a 2005 Toyota 4Runner. Accept 12 V through a fused sealed connector; include reverse-polarity and transient protection; regulate ESP32 power; provide two pressure interfaces, at least three temperature inputs, microSD, USB recovery, BLE/Wi-Fi, service button, LED, labeled test pads and optional CAN expansion. Preserve antenna keep-out. Design explicit self-test observability for VIN, regulated rails, sensor open/short detection and SD write/readback. Prefer JLCPCB-available parts. Generate schematic, routed PCB, BOM, CPL, Gerbers, STEP, DRC/ERC and machine-readable pin map. Flag automotive transient and refrigerant compatibility assumptions for engineering review.

BUILD PHASES

0 Bench ESP32-S3 + sensors + SD + BLE/AP.

1 Protected vehicle power + enclosure + harness + supported A/C interfaces.

2 Flux custom carrier PCB + ERC/DRC + JLCPCB PCBA.

3 Automated factory/self-test fixture + firmware provisioning.

4 iOS app + web dashboard + OTA.

5 OBD-II/CAN and maintenance telemetry.

6 Miniaturized coin-cell BLE pressure pucks after mechanical/refrigerant validation.

IMMEDIATE NEXT ACTIONS

1. Freeze V0 around ESP32-S3.

2. Select exact pressure transducers and obtain explicit R-134a/PAG compatibility confirmation.

3. Measure service-port clearance and design supported adapter geometry.

4. Order enclosure, connectors, TXL wire, loom, fuse hardware, standoffs, vibration mounts and SD hardware.

5. Bench-build sensor -> SD -> BLE -> Wi-Fi AP.

6. Implement POST/BIT before vehicle installation.

7. Create Flux project and issue the agent brief.

8. Design V1 for JLCPCB assembly.

9. Create manufacturing-package checker and factory JSON test.

10. Install first PCBA only after bench self-test passes.

CURRENT LINKS

ESP32-S3: https://www.espressif.com/en/products/socs/esp32-s3

TXL wire example: https://www.ictbillet.com/products/25ft-txl-automotive-grade-wire

R-134a coupler example: https://www.homedepot.com/p/XtremepowerUS-Auto-HVAC-R134A-A-C-Quick-Coupler-Adapter-Set-with-Low-and-High-Manifold-Connectors-26014/310878159

Flux AI PCB: https://www.flux.ai/p/nb/design-pcb-with-ai

Flux MCP: https://docs.flux.ai/reference/flux-mcp-server

JLCPCB API: https://api.jlcpcb.com/

JLCPCB PCBA requirements: https://jlcpcb.com/help/article/pcb-assembly-faqs

ESP32 Wi-Fi AP: https://docs.espressif.com/projects/arduino-esp32/en/latest/api/wifi.html

DEFINITION OF DONE

V1 is complete when the installed unit boots reliably from vehicle power, passes self-test, reads both pressure channels and temperatures, logs to SD, shows live data on iPhone over BLE, exposes the same data through its own Wi-Fi AP, recovers from a failed firmware update, produces a downloadable diagnostic report, and can be reproduced from a version-controlled manufacturing package.