PRODUCT REQUIREMENTS DOCUMENT

2005 Toyota 4Runner
Vehicle Health & Maintenance OS

Working product name: 4Runner Vehicle Health OS

PRD v0.1  •  Development Baseline  •  16 August 2026

Core principle

No unexplained number anywhere in the system.

Every health score, prediction and service recommendation must be traceable to evidence,
a versioned equation/rule, and permanent vehicle history.

# Document Control

| Field | Value |
| --- | --- |
| Document | Product Requirements Document — 2005 Toyota 4Runner Vehicle Health & Maintenance OS |
| Version | 0.1 — initial development baseline |
| Date | 16 August 2026 |
| Status | Ready to decompose into engineering epics and implementation tickets |
| Primary platform | In-vehicle Android device / infotainment display |
| Vehicle interface | DLC3 / OBD-II via custom ESP32 CAN gateway; existing Bluetooth OBD adapter may be used for bootstrap comparison |
| Vehicle scope | 2005 Toyota 4Runner; engine/drivetrain/trim-specific configuration resolved at onboarding; rear air-suspension support required |
| Connectivity posture | Local-first; internet optional; vehicle-bus access read-only by default |
| Data posture | Append-only evidence and service history; derived values reproducible and versioned |

Decision rule for this PRD When an exact Toyota PID, service interval, threshold or trim-specific value has not yet been verified against the target vehicle, this document defines the capability and validation workflow rather than inventing a vehicle-specific constant. Those constants live in versioned configuration packs.

# Table of Contents

1. Executive Summary

2. Product Vision and Principles

3. Goals, Non-Goals and Success Criteria

4. Vehicle and User Context

5. Product Scope and Release Boundaries

6. User Personas and Core Journeys

7. Information Architecture and UX

8. Functional Requirements

9. Maintenance Coverage Model

10. Evidence, Equations and Explainability

11. Data Model and Event Contracts

12. System Architecture

13. ESP32 Gateway and Signal Discovery

14. Android Application Architecture

15. AI Analysis Layer

16. Alerts and Driver Safety

17. Security, Privacy and OTA

18. Non-Functional Requirements

19. Test, Validation and Replay Strategy

20. Delivery Plan / Engineering Epics

21. MVP Acceptance Criteria

22. Risks and Open Decisions

23. Initial Backlog

24. Sources and Technical References

# 1. Executive Summary

4Runner Vehicle Health OS is an in-vehicle maintenance intelligence, diagnostics and digital-service-record platform for a 2005 Toyota 4Runner. It combines Toyota ECU/OBD data, optional added sensors, physical inspections and service records into a persistent digital twin of the specific vehicle. The primary UI runs on an internal Android infotainment device; a custom ESP32 gateway provides controlled access to the vehicle diagnostic/CAN interface and can store capture logs for offline replay.

The product is intentionally different from a conventional OBD dashboard. Live gauges are inputs, not the product. The product converts evidence into maintenance decisions: what is healthy, what is degrading, what is due, what should be inspected, why the system believes that, how confident it is, and what changed after a repair.

Maintenance-first: prioritize actionable component health and service state rather than maximizing the number of gauges.

Glass-box calculations: every derived result exposes its inputs, equation/rule version, timestamps, units, data-quality flags and confidence.

Permanent history: services, parts, inspections, faults, trends, trips, captures and baselines remain reconstructible years later.

Local-first reliability: core collection, storage, calculations and UI work without internet connectivity.

Safe vehicle access: MVP is passive/read-only on the vehicle bus. No throttle, brake, steering, suspension actuation, ECU reflashing or code clearing is automated.

Replayable development: capture bundles can be replayed against new decoders and equations, enabling rapid iteration without repeatedly experimenting on the live vehicle.

Evidence-bound AI: AI interprets deterministic evidence and may propose diagnostic next steps or gateway configuration changes, but it is not the source of truth and cannot autonomously control vehicle systems.

Figure 1. Reference product architecture.

# 2. Product Vision and Principles

## 2.1 Product Vision

Give a 2005 4Runner an auditable, continuously learning maintenance computer: the vehicle should be able to show its current state, explain how that state was calculated, preserve what happened to each component over time, and make the next maintenance action obvious.

## 2.2 Product Principles

| Principle | Requirement |
| --- | --- |
| P1 — No unexplained number | A displayed score, remaining-life estimate or recommendation must link to the exact calculation/evidence that generated it. |
| P2 — Measurement and estimate are visibly different | Physical measurements, ECU-reported values, inferred condition, schedule-based due dates and manual inspections must never be visually conflated. |
| P3 — History is append-only | A new model may recalculate history, but the original historical result remains preserved with the original equation version. |
| P4 — Confidence is first-class | Every inferred result carries confidence and the reasons for reduced confidence: stale inspection, missing signals, uncertain decoder, noisy sensor or unvalidated model. |
| P5 — Read-only until proven safe | The product gathers data without changing vehicle behavior. Any future actuation requires a separate safety case and separate product scope. |
| P6 — Offline operation is normal | A loss of internet must not stop collection, maintenance calculations, alerts, history or core UI. |
| P7 — Vehicle-specific learning | Baselines are built for this individual 4Runner and reset/annotated when the relevant component is serviced or replaced. |
| P8 — Reproducible experiments | Signal discovery, decoder changes and model changes are tested against versioned capture bundles and golden datasets. |

# 3. Goals, Non-Goals and Success Criteria

## 3.1 Goals

Collect and normalize all maintenance-relevant data available from the target 2005 4Runner ECUs plus approved external sensors.

Maintain a component-level digital twin and service ledger with mileage, time, operating-hours and condition history.

Calculate schedule-based, usage-adjusted and condition-estimated maintenance states with explicit confidence.

Provide a driving-safe summary UI and a parked “Garage / Engineering” UI with deep drill-down.

Allow any recommendation to be traced to evidence and equation/rule lineage.

Provide a reproducible protocol/PID discovery loop for the ESP32 gateway and Android analysis stack.

Support secure, rollback-capable ESP32 firmware/configuration updates initiated from the phone/Android side.

Enable an AI agent to interpret evidence, compare historical behavior and propose diagnostic or decoder next steps without becoming the authoritative calculator.

Export a complete vehicle health/service package for backup, mechanic handoff or future migration.

## 3.2 Non-Goals for MVP

No autonomous vehicle control or safety-critical actuation.

No ECU firmware flashing, immobilizer programming or emissions-system modification.

No claim that a modeled remaining-life percentage is equivalent to physical inspection unless a calibrated measurement exists.

No cloud dependency for core collection or calculations.

No universal support for every Toyota model in the first release; architecture should be extensible, but validation targets the specific 2005 4Runner first.

No automatic purchase of parts or booking of service.

No hidden AI-generated maintenance score.

## 3.3 Product Success Criteria

| Area | Success criterion |
| --- | --- |
| Traceability | 100% of displayed derived metrics open an Evidence / Calculation view containing source IDs and calculation version. |
| Data durability | No acknowledged service event or accepted inspection is lost across reboot, app update or gateway update. |
| Offline behavior | Core dashboard, history, alerts and calculation engine operate with network disabled. |
| Replayability | A recorded capture can be replayed through a newer decoder/model and produces a separately versioned calculation run. |
| Reconnect reliability | Gateway disconnect/reconnect does not require an app restart and does not duplicate persisted samples. |
| Safety posture | Vehicle-bus transmission is blocked by default outside a narrowly defined diagnostic-request allowlist; no Active Test/control commands in MVP. |
| Explainability | A user can reach raw evidence from a top-level health card in no more than four taps while parked. |
| Maintenance completeness | Every maintenance item in the configured Toyota schedule has a tracked state even if its condition can only be established by manual inspection. |

# 4. Vehicle and User Context

## 4.1 Target Vehicle

Initial deployment target: 2005 Toyota 4Runner with rear air suspension. The product must not assume the engine, drivetrain or trim until the vehicle profile is resolved. The configuration system shall support the model-year variants relevant to service rules (for example V6 vs V8 and 2WD vs 4WD), because maintenance schedules and accessible ECU data can differ.

## 4.2 Diagnostic Interface Facts and Assumptions

Toyota repair information mirrored for the 2005 4Runner states that the ECM communicates through DLC3 using ISO 15765-4 CAN [S2].

The ESP32 family provides a TWAI (CAN-compatible) controller that requires an external transceiver; current ESP-IDF exposes receive, alert and bus-recovery facilities [S3].

Toyota-specific Data List items beyond generic OBD-II are expected to require an explicit registry/decoder rather than assuming generic Mode 01 coverage.

Exact ECU addresses, supported diagnostic services, enhanced PIDs, update rates and scaling shall be enumerated from the target vehicle and stored in a versioned Vehicle Signal Pack.

The existing Bluetooth OBD adapter may be used as an independent baseline to compare requests/responses during bootstrap, but production architecture should not depend on an opaque third-party adapter.

## 4.3 Primary User Roles

| Role | Primary need |
| --- | --- |
| Owner / driver | Wants a simple answer: healthy, due, developing issue, inspect, service now; minimal distraction while driving. |
| Garage / DIY maintainer | Needs service history, part records, inspection measurements, maintenance queues and evidence. |
| Engineer / developer | Needs raw frames, signal registry, decoder versions, captures, replay, experiment results, equation definitions and firmware status. |
| AI diagnostic agent | Read-only consumer of structured evidence; can summarize, correlate and propose next tests/config changes; cannot silently alter authoritative history or control the vehicle. |
| Mechanic / external reviewer | Receives an export containing service records, recent findings, DTCs, measurements and evidence summaries without needing the full application. |

# 5. Product Scope and Release Boundaries

| Release | Scope |
| --- | --- |
| Phase 0 — Instrumentation bootstrap | Android shell; local database; synthetic/replay data source; existing OBD adapter comparison; ESP32 read-only capture; capture-bundle format; signal-registry tooling. |
| MVP — Maintenance ledger + explainable health | Vehicle profile; core ECU signals; maintenance schedule engine; parts/service records; equation registry; calculation runs; dashboard; maintenance queue; component pages; history; DTC/event ingestion; gateway status; offline export. |
| V1 — Vehicle-specific condition intelligence | Enhanced Toyota signal pack; tire/TPMS integration; rear air-suspension analytics; transmission thermal model; battery/crank trends; fuel-trim/misfire analytics; manual inspection calibration; AI evidence assistant. |
| V1.5 — Added sensor network | A/C pressure sensors, current/voltage sensing, differential/transfer-case temperature, air-system pressure, IMU/vibration; sensor provisioning and calibration. |
| V2 — Fleet/generalization | Reusable Toyota/vehicle configuration packs, remote backup/sync, mechanic portal, broader model support. Any control/actuation remains a separate safety-reviewed program. |

# 6. User Personas and Core Journeys

| Journey | Happy path |
| --- | --- |
| J1 — Morning status | Driver starts vehicle → Android connects to gateway → current health summary appears → only urgent/critical items interrupt → non-urgent due items remain in queue. |
| J2 — “Why is suspension 76?” | Parked user taps Suspension → sees health contributors → taps decreasing score → opens calculation → taps input → sees historical trend → taps evidence → sees source signal/capture and timestamp. |
| J3 — Perform service | User opens due item → service wizard pre-fills component, mileage and evidence → enters part/fluid/quantity/cost/notes/attachments → confirms service → relevant lifecycle closes → new component/service baseline begins. |
| J4 — Physical inspection | System says “inspection due; condition unknown” → user enters measured pad thickness / belt condition / fluid state → confidence increases and model recalibrates. |
| J5 — Developing fault | Trend engine detects abnormal tire pressure decay or compressor fill-time increase → finding created with confidence and evidence → user is notified → finding can be resolved, dismissed with reason or linked to a service event. |
| J6 — Decoder iteration | Gateway capture fails or a desired Toyota signal is unknown → capture bundle synced to Android → replay/analysis scores candidate decoder/config → agent proposes next experiment → user/safety gate approves → signed gateway config/firmware deployed → next capture compared to previous. |
| J7 — Firmware update | Android downloads/builds approved signed ESP32 image → checks parked/update preconditions → transfers to inactive OTA slot → gateway self-tests → marks valid or rolls back automatically [S4][S5]. |
| J8 — Mechanic handoff | User exports vehicle report → contains profile, open findings, DTC history, recent graphs, service ledger, parts and confidence/evidence summaries → no account required to read export. |

# 7. Information Architecture and UX

## 7.1 Top-Level Navigation

| Destination | Contents |
| --- | --- |
| Home | Vehicle health, critical/urgent items, upcoming service, developing conditions, gateway status. |
| Maintenance | Due / upcoming / inspect / completed; schedule and condition views. |
| Vehicle | System/component tree and installed-part digital twin. |
| History | Unified timeline of service, parts, inspections, DTCs, findings, trips, firmware and calculations. |
| Signals | Live signal explorer and charts; Garage mode only for deep detail. |
| Diagnostics | DTCs, findings, capture sessions, health checks, experiment/replay tools. |
| Garage | Service-entry, inspection-entry, parts, attachments, calibration, exports. |
| Settings | Vehicle profile, units, data retention, connectivity, gateway, firmware, AI, developer options. |

## 7.2 Home Screen

Overall vehicle state must not pretend to be a single scientifically exact “health percentage.” If an aggregate score is shown, it is secondary and fully decomposable into component states.

Primary cards: Engine, Transmission, Cooling, Brakes, Tires, Suspension, Electrical, HVAC, Driveline, Scheduled Maintenance.

Every card shows state, trend arrow, confidence badge and primary action. Example states: Healthy, Watch, Inspect, Due Soon, Due, Urgent, Critical, Unknown.

Display “Unknown / inspection required” instead of estimating condition without evidence.

Driver mode suppresses graphs, raw signals, engineering controls and long text. Parked Garage mode exposes deep navigation.

## 7.3 Component Detail

Current state and confidence.

Latest direct measurements and most important derived metrics.

Trend over selectable windows: trip / 7 days / 30 days / 90 days / since installation.

Installed part/fluid record, age, mileage and operating hours.

Open findings and maintenance recommendations.

Last physical inspection and next inspection due.

Buttons: Explain, Evidence, History, Service, Add inspection, Live signals.

## 7.4 Equation Inspector

Human-readable equation name and purpose.

Stable equation ID and immutable version.

Equation expression, unit contract, applicability and validation status.

Exact inputs used for this calculation run, each with value, unit, source, timestamp and quality flag.

Intermediate calculations and final output.

Confidence calculation and missing/weak evidence reasons.

Link to previous equation version and optional “recalculate with current version” action that creates a new run without overwriting history.

Figure 2. Required evidence lineage.

## 7.5 Unified Timeline

The timeline is the chronological truth layer for the vehicle. It must support filters by system/component/event type and must never silently delete or rewrite accepted service history. Corrections are represented as amendments linked to the original event.

Service / repair

Inspection

Installed or removed part

DTC appeared / cleared externally

Diagnostic finding opened / resolved

Alert

Trip

Firmware / configuration change

Signal pack / equation version change

Calibration / baseline reset

Attachment / receipt

Data gap / gateway outage

# 8. Functional Requirements

| ID | Pri | Requirement | Acceptance / testable outcome |
| --- | --- | --- | --- |
| FR-001 | P0 | Create a vehicle profile containing VIN (when available), model year, engine, drivetrain, trim/options, tire configuration, rear air-suspension presence, odometer source and unit preferences. | Profile persists across reboot; engine/drivetrain may be Unknown until resolved; engine-specific rules cannot activate while applicability is unresolved. |
| FR-002 | P0 | Support a versioned Vehicle Configuration Pack that defines ECU inventory, signal definitions, diagnostic requests, scaling, units, maintenance schedule applicability and known component mappings. | Pack version is recorded on every derived calculation that depends on it. |
| FR-003 | P0 | Connect Android to the ESP32 gateway over BLE as a supported baseline transport; architecture shall allow Wi-Fi/USB alternatives. | Connection status is visible; disconnect/reconnect does not require process restart; transport is abstracted from signal processing. |
| FR-004 | P0 | Ingest timestamped raw gateway frames and decoded diagnostic responses. | Every sample has source, monotonic capture time, wall time when available, sequence number and quality status. |
| FR-005 | P0 | Store raw capture bundles independently of decoded values. | A capture can be replayed after decoder updates without re-driving the vehicle. |
| FR-006 | P0 | Maintain a signal registry with stable IDs, ECU/source, address/PID/service, byte layout, endian/sign, scaling equation, unit, range, cadence, applicability and confidence. | Registry changes create a new version; old captures remain decodable with their original version. |
| FR-007 | P0 | Normalize decoded signals into canonical units while retaining original raw bytes/value. | UI unit changes do not alter persisted canonical values or raw source evidence. |
| FR-008 | P0 | Assign quality flags to signals: good, stale, missing, out-of-range, decoder-uncertain, transport-gap, sensor-unverified, manually-entered. | Derived calculations surface any non-good inputs. |
| FR-009 | P1 | Group data into trips/sessions using ignition/engine-run evidence with graceful fallback when a source is missing. | Each sample may be traced to a trip/session; ambiguous session boundaries are marked, not guessed silently. |
| FR-010 | P0 | Ingest odometer/mileage and maintain a mileage truth model with source priority and manual correction workflow. | Mileage corrections create an amendment; no retroactive silent rewrite. |
| FR-011 | P0 | Represent the Toyota maintenance schedule as data, not UI logic. | Schedule rules can be updated by configuration pack and filtered by vehicle applicability/severe-use criteria. |
| FR-012 | P0 | Calculate due state using mileage and time, whichever is earlier when the source schedule requires both. | Displayed due reason states whether mileage, time or both caused the state. |
| FR-013 | P1 | Track usage-severity features such as towing/off-road/short-trip/idle/temperature exposure when evidence exists. | Severity adjustment must show contributing features and configurable model version; unavailable evidence does not default to “severe.” |
| FR-014 | P0 | Create a component hierarchy and lifecycle record for maintainable systems/components. | Component record exposes installed part, current lifecycle start, service/inspection history, findings and relevant signals. |
| FR-015 | P0 | Record installed parts/fluids with manufacturer, part number/spec, quantity, cost, source, date, mileage, notes and attachments. | A replacement closes the prior lifecycle and starts a new lifecycle without deleting prior history. |
| FR-016 | P0 | Record maintenance events and inspections with structured measurements. | Entries are append-only; corrections are amendments; each entry records author/source and time. |
| FR-017 | P0 | Allow photo, receipt/document and free-note attachments to service/inspection records. | Attachment metadata and hash persist; missing external file is visibly flagged. |
| FR-018 | P0 | Maintain an immutable Equation Definition registry with stable ID, version, expression, inputs, units, applicability, thresholds/constants, validation status and release notes. | A modified expression creates a new version; calculations never point to mutable equation text. |
| FR-019 | P0 | Persist a Calculation Run for every user-visible derived metric snapshot or finding trigger. | Run stores equation version, exact inputs/evidence refs, intermediates, output, confidence and execution timestamp. |
| FR-020 | P0 | Support deterministic recalculation of a historical run using the original equation/config version. | Recomputed value matches within defined numerical tolerance or raises a reproducibility error. |
| FR-021 | P1 | Support “recalculate with current model” without overwriting historical output. | New run links to original run and indicates model-version delta. |
| FR-022 | P0 | Separate manufacturer schedule state, usage-adjusted state, condition-estimated state and physical inspection state. | UI labels each state explicitly and does not collapse unknown inspection condition into a health percentage. |
| FR-023 | P0 | Create Diagnostic Findings as structured objects with suspected component(s), severity, confidence, evidence refs, triggering rule/model and recommended next action. | Finding can be resolved by service, inspection, evidence disappearance or explicit dismissal with reason. |
| FR-024 | P0 | Maintain DTC history including first seen, last seen, count, status and source ECU when available. | Clearing externally does not erase historical occurrence. |
| FR-025 | P0 | Provide a maintenance queue sorted by urgency, due state and confidence. | User can filter by due/inspect/watch/completed/system and open evidence from each item. |
| FR-026 | P0 | Provide a driving-safe Home view. | While vehicle is moving, no raw frame controls, firmware actions or complex drill-down actions are shown. |
| FR-027 | P0 | Provide parked Garage/Engineering mode. | Deep signal/equation/history/diagnostic tools are available when parked or developer override is intentionally enabled for bench use. |
| FR-028 | P0 | Provide a Signal Explorer with search, live value, min/max, sparkline/history, source, rate and quality. | User can trace a signal to registry definition and source captures. |
| FR-029 | P0 | Provide Equation Inspector and Evidence Trace views. | From any derived metric, user reaches equation inputs and source evidence in <=4 taps while parked. |
| FR-030 | P0 | Provide component-specific history charts from installation/service baseline. | Chart indicates data gaps, service/reset markers and equation-version changes. |
| FR-031 | P1 | Allow user-defined dashboards from registered signals/metrics without affecting authoritative maintenance rules. | Removing a widget cannot delete underlying data. |
| FR-032 | P0 | Support configurable alert severities and deduplication/hysteresis. | Repeated identical state does not spam alerts; transitions and sustained conditions are logged. |
| FR-033 | P0 | Critical warnings require explicit deterministic criteria and high-quality evidence. | If evidence quality is insufficient, severity degrades to “inspect/watch” rather than presenting a false critical assertion. |
| FR-034 | P1 | Support per-tire records and TPMS pressure/temperature where decoded/available; support rotation events that remap physical tire to wheel position. | History follows the physical tire, not just LF/RF/LR/RR position. |
| FR-035 | P1 | Track TPMS sensor/transmitter lifecycle where IDs/battery condition or replacement history are available. | Sensor ID and service record remain linked after tire rotation. |
| FR-036 | P1 | Support rear air-suspension analytics: height response, compressor runtime/cycles, parked height decay and added compressor-current/pressure data when available. | Health finding shows which inputs are direct vs inferred and uses a component-specific baseline. |
| FR-037 | P1 | Support transmission thermal exposure and shift/slip observations when accessible. | Thermal model exposes integration threshold/constants and data coverage; missing ATF temperature prevents condition claim. |
| FR-038 | P1 | Support battery/charging/crank trend analysis from voltage and optional current sensing. | Crank events are detected reproducibly and poor sampling lowers confidence rather than inventing state. |
| FR-039 | P1 | Support fuel-trim, O2/A-F, misfire and catalyst/EVAP observations where signals/monitors exist. | Findings distinguish ECU monitor/DTC facts from higher-level inference. |
| FR-040 | P1 | Support brake inspection measurements and a calibrated wear model using cumulative usage features after at least one physical measurement. | UI shows last measured thickness separately from estimated current thickness. |
| FR-041 | P1 | Support fluid lifecycle tracking for engine oil, coolant, ATF, transfer case, differentials, brake fluid and power steering when applicable. | Each fluid can have schedule, usage, condition/temperature evidence and service spec/quantity record. |
| FR-042 | P1 | Support added sensor provisioning, calibration, battery state and mapping to components. | Uncalibrated sensor cannot be treated as authoritative good-quality evidence. |
| FR-043 | P0 | Export an offline vehicle package containing profile, configuration versions, service ledger, parts, inspections, open findings, DTC summary, selected trends and evidence manifest. | Export is self-describing and checksum-verifiable; user can choose to exclude raw high-volume captures. |
| FR-044 | P1 | Import a prior export/backup without duplicating events. | Import uses stable IDs/checksums and reports conflicts instead of silently overwriting. |
| FR-045 | P0 | Expose gateway firmware, config pack, boot status, uptime, reset reason, storage and transport health. | Gateway health is distinguishable from vehicle health. |
| FR-046 | P0 | Allow signed gateway OTA update with rollback/self-test. | Update never writes vehicle ECUs; failed self-test rolls gateway back to prior valid image/config [S4][S5]. |
| FR-047 | P0 | Block firmware update while unsafe preconditions exist (for example active capture requiring continuity, insufficient gateway power margin, or moving vehicle). | UI states failed precondition; no partial update begins. |
| FR-048 | P1 | Store gateway diagnostic reports/capture bundles on Android and make them available to the analysis agent. | Agent receives immutable bundle ID, firmware/config versions and experiment manifest. |
| FR-049 | P1 | Create Discovery Experiments with hypothesis, candidate protocol/config, capture window, expected success signals, metrics and outcome. | Experiment can be replayed and compared to another experiment using common metrics. |
| FR-050 | P1 | Allow AI/software agent to propose the next discovery configuration or decoder change. | Proposal is a reviewable diff and cannot directly bypass vehicle-bus allowlist or OTA safety gate. |
| FR-051 | P1 | Provide a replay engine for captures into decoder, feature and rule layers at controllable speed. | Replay output is isolated from live vehicle alerts unless explicitly run in test namespace. |
| FR-052 | P1 | Support a simulator/synthetic signal source for UI and automated tests. | Entire app can run without a connected vehicle/gateway. |
| FR-053 | P0 | Audit every authoritative mutation: service entry, inspection, amendment, configuration activation, equation activation, firmware update and manual override. | Audit record contains actor/source, timestamp, prior/new reference and reason when applicable. |
| FR-054 | P1 | Support local AI and/or remote AI provider through a provider abstraction. | Core product works with AI disabled; no raw data leaves device unless explicitly configured. |
| FR-055 | P1 | Require evidence citations in AI-generated diagnostic statements shown as factual vehicle claims. | Each vehicle-condition sentence can be expanded to supporting internal evidence IDs; unsupported claims are labeled as hypothesis/general guidance. |
| FR-056 | P0 | Persist data through Android reboot and automatically restore intended gateway connection behavior within Android platform limits. | App recovers without data corruption; background connection implementation follows current Android BLE/background restrictions [S6][S7]. |
| FR-057 | P1 | Support data-retention tiers: dense raw frames, downsampled signals, derived metrics, permanent service/inspection history. | Retention policy can purge eligible raw data without deleting evidence referenced by preserved findings unless safely compacted with hash/reference. |
| FR-058 | P0 | Provide units and timezone handling that does not corrupt canonical timestamps/measurements. | Stored time is unambiguous; displayed local time/unit changes are presentation-only. |
| FR-059 | P0 | Detect and display stale/missing data rather than holding the last value as live. | Every live card has source freshness logic. |
| FR-060 | P0 | Record system/model version metadata with findings and exported reports. | A report can state exactly which app, signal pack, equations and gateway firmware produced it. |

# 9. Maintenance Coverage Model

The system must track every scheduled or practically maintainable item even when no electronic signal directly measures its condition. Each item is assigned one or more evidence modes: OEM schedule, ECU/direct, derived, added sensor, physical inspection, service record. “Condition unknown” is a valid state.

| Maintenance item | Evidence sources | System output | Truth boundary |
| --- | --- | --- | --- |
| Engine oil | Schedule + mileage/time + operating severity; optional oil temp/pressure | Oil/filter due; usage-adjusted interval; overheat/short-trip severity | Oil quality is estimated unless sampled/sensed. |
| Oil filter | Schedule + oil service record | Due with oil; service lifecycle | No direct filter restriction sensor. |
| Engine coolant | Coolant temp + schedule + service record; optional level sensor | Warm-up/cooling trends, overheat history, service due | Fluid chemistry/contamination requires inspection/test. |
| Thermostat / cooling efficiency | Coolant + ambient/intake temp + load + speed | Warm-up drift / thermal-control anomaly | Diagnosis is probabilistic; inspect before replacement. |
| Water pump / radiator / hoses | Cooling trends + inspection + added leak/level evidence | Leak/thermal anomaly; inspection schedule | Visual leakage/play remains inspection-based. |
| Engine air filter | MAF/load trends + mileage + environment + inspection | Restriction proxy / service due | Physical inspection preferred before replacement. |
| Cabin air filter | Time/mileage/HVAC usage + manual inspection | Due reminder | No OEM differential-pressure sensor. |
| Spark plugs | Schedule + misfire/fuel-trim/ignition behavior + service record | Due / abnormal combustion clues | Plug condition requires inspection. |
| Ignition coils | Misfire patterns + DTCs + service history | Cylinder-specific anomaly | Do not equate misfire with failed coil without corroboration. |
| Fuel system / injectors | Fuel trims, misfires, O2/A-F, load; optional pressure sensor | Mixture/injector imbalance clues | Fuel pressure requires supported PID or added sensor. |
| O2 / A-F sensors | ECU sensor data + DTC/monitor history | Response/bias anomaly, replacement lifecycle | Toyota-specific signal mapping required. |
| Catalytic converters | ECU monitor/DTC + upstream/downstream behavior | Monitor status / degradation clues | Do not bypass emissions logic. |
| EVAP | Readiness/monitor/DTC + available pressure/valve data | Leak/system finding | No actuation Active Tests in MVP. |
| Timing belt / chain | Engine-specific schedule + verified service history | Hard due/unknown-history action | No reliable remaining-life sensor; engine applicability critical. |
| Accessory drive belt | Schedule/time + inspection | Inspect/replace due | Cracks/tension require physical inspection unless added vision/sensor. |
| Battery | Voltage, crank sag/duration, time/temperature; optional current | Cranking/aging trend | Battery chemistry/CCA test may be entered manually. |
| Alternator / charging | Running voltage; optional current/ripple sensing | Charge instability / output trend | Accessory loads influence model. |
| Starter | Crank duration/RPM/voltage; optional current | Slow-crank trend | Separate battery vs starter with confidence. |
| ATF / transmission | ATF temp if available, schedule, shifts/slip, service record | Thermal stress, service due, shift anomaly | Fluid contamination still requires inspection/sample. |
| Transfer case fluid | Schedule + 4WD usage + towing/off-road + service record; optional temp | Due / severity-adjusted usage | No OEM fluid-quality sensor. |
| Front differential fluid | Schedule + 4WD/off-road/tow + service record; optional temp | Due / thermal severity | Fluid condition inspection-based. |
| Rear differential fluid | Schedule + load/tow/off-road + service record; optional temp | Due / thermal severity | Fluid condition inspection-based. |
| Propeller shaft / U-joints | Mileage/use + inspection/service record + optional vibration | Lube/inspect due; vibration anomaly | Play/grease state requires inspection. |
| CV axles / boots | 4WD/use + inspection + optional vibration | Inspect; anomaly trend | Boot tears are visual. |
| Wheel bearings | Wheel-speed consistency + optional IMU/acoustic + inspection | Anomaly suspicion | Final confirmation requires physical diagnosis. |
| Brake pads | Inspection thickness + braking usage/energy model | Measured thickness + estimated wear | Estimate never replaces measured thickness. |
| Brake rotors | Inspection measurements + brake vibration/usage | Inspect/replace due; anomaly | Thickness/runout require measurement. |
| Brake fluid | Schedule/time + reservoir warning + manual moisture test | Service due / low-level alert | Moisture/boiling point entered from test. |
| ABS / VSC | DTCs + wheel speed + supported chassis signals | System fault / sensor anomalies | No Active Test actuation in MVP. |
| Power steering | Service/inspection + available load/engine changes; optional temp | Leak/condition service record | Fluid level/condition normally inspection-based. |
| Tire pressure | TPMS pressure where decoded or external TPMS | Low pressure, leak rate, pressure history | Temperature compensation required for leak analytics. |
| Tire temperature | TPMS temperature where available | Thermal imbalance / context for pressure | Availability must be verified on target vehicle. |
| Tire wear / rotation | Tire lifecycle + rotation history + manual tread depth | Rotation due; wear rate; position-specific wear | Tread depth manual/vision measurement. |
| TPMS transmitters | Sensor IDs + history + available status | Sensor lifecycle / missing transmitter | Battery condition may be inferred only if supported. |
| Alignment | Steering/wheel-speed trends + tire wear measurements | Drift suspicion | Alignment angles require alignment rack/manual values. |
| Rear air springs | Height sensors + parked decay + compressor behavior | Leak/degradation trend | Temperature/load normalization needed. |
| Air suspension compressor | Compressor command/runtime + height response; optional current/pressure | Duty-cycle / efficiency degradation | Use component baseline and service reset. |
| Height sensors / control | Height values + cross-side consistency + DTCs | Sensor/control anomaly | No automatic actuation tests. |
| Shocks / bushings / ball joints / tie rods | Mileage/use + physical inspection + optional IMU | Inspect due / vibration clues | Wear/play is inspection-based. |
| A/C refrigerant | HVAC behavior + added high/low pressure + ambient | Charge/performance trend | Added pressure sensors are required for pressure-based diagnosis. |
| A/C compressor | Command/load + pressures + optional current/temp | Efficiency/anomaly trend | Avoid claiming failure from one signal. |
| Exhaust | O2/fuel trim/catalyst clues + inspection | Leak/catalyst clues | Physical leak confirmation required. |
| Frame / corrosion | Inspection/photos + environment/history | Inspection due; condition record | No electronic OEM measurement. |
| Wipers / washer / lights | Time/service record + supported switch/status signals | Replace/check reminders | Condition primarily visual/manual. |

# 10. Evidence, Equations and Explainability

## 10.1 Lineage Contract

Every authoritative derived value must carry a complete lineage graph. A calculation may use multiple signals and prior calculations; each edge is persisted by stable ID. If the system cannot reconstruct a value, the value cannot be treated as authoritative.

RawObservation -> SignalSample -> FeatureValue -> CalculationRun -> DerivedMetric/Finding -> Recommendation -> MaintenanceEvent -> BaselineReset

## 10.2 Equation Definition Schema

| Field | Requirement |
| --- | --- |
| equation_id | Stable semantic ID, e.g. tire.leak_rate or suspension.compressor_efficiency. |
| version | Immutable semantic version. |
| expression | Executable deterministic expression or rule graph. |
| human_explanation | Plain-language description shown in Equation Inspector. |
| inputs | Typed variables with required units, freshness and quality constraints. |
| constants | Named thresholds/weights with provenance and vehicle applicability. |
| outputs | Typed unit/range and interpretation. |
| confidence_model | How input quality, coverage, recency and validation affect confidence. |
| applicability | Vehicle/engine/system conditions required for use. |
| validation_status | Experimental / bench validated / vehicle validated / calibrated for this vehicle. |
| release_notes | Why the version changed and expected behavior delta. |

## 10.3 Seed Equation Library (Development Models)

The formulas below are product-framework seeds, not Toyota-certified service specifications. Thresholds and weights are configuration data and must be validated against service information, measurements and target-vehicle captures before they drive high-severity recommendations.

| Model | Example expression | Design requirement |
| --- | --- | --- |
| Schedule due ratio | due_ratio = max(miles_since_service / interval_miles, days_since_service / interval_days) | Tracks which schedule dimension is controlling; supports one-dimensional rules by omitting absent denominator. |
| Usage-equivalent miles | equiv_miles = miles * (1 + w_tow*f_tow + w_offroad*f_offroad + w_short*f_short + w_idle*f_idle + w_heat*f_heat) | Severity features and weights are visible/configurable; use only when evidence exists. |
| Temperature-corrected tire pressure | P_ref_abs = P_abs * T_ref_K / T_K; P_ref_gauge = P_ref_abs - P_atm | Normalizes pressure for temperature before leak-rate regression; atmospheric-pressure approximation/source must be recorded. |
| Tire leak rate | leak_rate = slope(least_squares(time_days, P_ref_gauge)) | Window, sample count, residual error and confidence persisted. |
| ATF thermal exposure | thermal_stress = integral(max(0, T_atf - T_ref) * dt) | T_ref is configuration, not a universal failure threshold; units degree-hours. |
| Warm-up drift | warmup_delta = observed_time_to_target - expected(ambient_temp, start_temp, route/load class) | Model is vehicle-specific baseline; large route differences lower confidence. |
| Fuel-trim load | trim_bias = weighted_mean(abs(STFT + LTFT), by operating cell) | Analyze across operating cells rather than a single idle sample. |
| Misfire density | misfire_density = misfire_count / normalized_engine_cycles | Per-cylinder when source supports it; DTC/monitor facts remain separately stored. |
| Crank voltage proxy | crank_score = F(V_min, crank_duration, rpm_rise, ambient_temp, baseline_distribution) | F is versioned and calibrated from this vehicle; voltage-only analysis cannot uniquely identify battery vs starter. |
| Suspension height-decay rate | height_decay = slope(time_hours, normalized_height_mm) while parked | Exclude load changes/door events and account for temperature when evidence exists. |
| Compressor efficiency | efficiency = delta_height_mm / compressor_runtime_s; optional electrical_eff = delta_height / (runtime * current) | Track baseline since compressor/air-spring service. |
| Brake thickness estimate | t_est = t_measured_last - k_vehicle * cumulative_brake_energy_since_measurement | k_vehicle is learned/calibrated from physical inspections; estimated and measured thickness shown separately. |
| Confidence | C = geometric_mean(source_quality, freshness, coverage, model_validation, calibration_quality) | Each factor [0,1] and displayed; exact aggregation may be equation-version specific. |

## 10.4 Calculation Run Requirements

calculation_run: id: calc_... metric_id: suspension.rear_health equation: AIR_SUSPENSION_HEALTH@1.4.0 vehicle_config_pack: 2005_4runner_target@0.3.2 inputs: - signal_ref: compressor.fill_time value: 10.81 unit: s evidence_id: obs_... - metric_ref: parked.height_decay value: 1.38 unit: mm/h calculation_id: calc_... output: 0.7646 unit: ratio confidence: 0.91 quality_notes: [] executed_at: 2026-08-16T10:41:17-04:00

# 11. Data Model and Event Contracts

| Entity | Key contents |
| --- | --- |
| Vehicle | vehicle_id, VIN, year/make/model, engine, drivetrain, trim/options, odometer policy, unit prefs |
| ECU | ecu_id, logical name, address, protocol, software/calibration identifiers if available, first/last seen |
| SignalDefinition | signal_id, registry_version, ECU/source, request/address, raw layout, transform, units, range, cadence, applicability |
| RawObservation | observation_id, capture_id, bus/source, raw payload, timestamps, sequence, gateway firmware/config, hash |
| SignalSample | sample_id, signal_id, observation_ref, canonical value/unit, source time, quality, trip/session |
| Trip / Session | session_id, start/end, mileage delta, engine hours, route/usage summary, data coverage |
| Component | component_id, taxonomy path, vehicle applicability, maintenance policies |
| InstalledPart | part_lifecycle_id, component_id, part/spec/brand, installed date/mileage/hours, removed date/mileage/hours, attachments |
| MaintenancePlan | plan_id, source/provenance, applicability, interval/rule expression, severe-use modifiers |
| MaintenanceEvent | event_id, component(s), action, date/mileage/hours, parts/fluids, cost, notes, attachments, actor |
| Inspection | inspection_id, component, method, measurements, units, condition grade, date/mileage, attachments |
| EquationDefinition | equation_id/version, expression/rule graph, inputs/outputs, constants, applicability, validation status |
| CalculationRun | calculation_id, equation version, config versions, evidence refs, exact inputs/intermediates/output/confidence |
| DerivedMetric | metric_id, latest calculation ref, component, state, trend |
| DiagnosticFinding | finding_id, type, suspected components, severity, confidence, evidence/calculation refs, lifecycle/state |
| Alert | alert_id, finding/maintenance ref, severity, trigger state, acknowledge/dismiss data |
| CaptureBundle | capture_id, manifest, raw log segments, gateway health, vehicle state, hashes |
| DiscoveryExperiment | experiment_id, hypothesis, config diff, capture refs, scoring, outcome, proposed next action |
| FirmwareBuild | firmware_id, git/build metadata, signature/hash, compatible hardware, OTA state, validation result |
| Attachment | attachment_id, type, content hash, local path/object ref, linked entities, metadata |
| AuditEvent | audit_id, actor/source, mutation type, before/after refs, timestamp, reason |

## 11.1 Canonical Signal Event

{ "event_type": "signal.sample.v1", "vehicle_id": "veh_...", "session_id": "trip_...", "signal_id": "engine.coolant_temp", "value": 88.4, "unit": "degC", "observed_at_monotonic_us": 38192019384, "wall_time": "2026-08-16T11:22:41.223-04:00", "quality": "GOOD", "source": {"ecu":"ECM", "observation_id":"obs_..."}, "versions": {"gateway":"0.7.1", "signal_pack":"0.3.2"} }

## 11.2 Capture Bundle Manifest

Capture ID, start/end, vehicle profile snapshot, odometer, ignition/engine state, stationary/moving flag.

Gateway hardware revision, firmware Git SHA/build ID, active config, CAN/TWAI settings, transport settings.

Raw log file list with hashes and byte counts.

Diagnostic requests issued (allowlisted only) and response statistics.

Bus error/overrun/drop counters, TWAI alerts, reconnect/reset reasons.

Decoder/signal-pack version, experimental configuration, hypothesis and operator notes.

Optional labeled events (“engine start”, “brake applied”, “suspension height change”, etc.) for supervised signal discovery.

# 12. System Architecture

## 12.1 Architectural Layers

| Layer | Responsibility |
| --- | --- |
| Vehicle interface | DLC3 connector, automotive-grade protection and CAN transceiver. No direct ESP32 GPIO connection to CANH/CANL. |
| Gateway firmware | TWAI receive/allowlisted diagnostic request, timestamping, buffering, microSD optional, health counters, BLE/Wi-Fi/USB transport, OTA, capture bundles. |
| Android ingest | Connection management, message validation, clock alignment, deduplication, raw observation persistence. |
| Signal/feature engine | Vehicle Signal Pack decoder, canonical units, quality/freshness, feature extraction, trip/session aggregation. |
| Maintenance/calculation engine | Schedule policies, equation registry/runtime, calculation-run persistence, confidence and finding generation. |
| Data services | Local structured DB + raw capture storage + attachments + export/backup. |
| Presentation | Compose UI, driving/garage policies, charts, timeline, signal explorer, equation inspector. |
| AI analysis | Read-only structured evidence interface, hypotheses/summaries, discovery experiment proposal and report generation. |

## 12.2 Local-First Data Flow

1. Gateway timestamps and forwards raw observations / decoded diagnostic responses.

2. Android validates envelope, sequence and source; persists raw evidence before or atomically with derived signal sample.

3. Signal pack converts bytes to canonical signal values and quality states.

4. Feature engine computes rolling/trip/lifecycle features.

5. Maintenance engine evaluates schedule/rules and executes deterministic equations.

6. Calculation runs and findings are persisted before UI alerting.

7. UI reads materialized latest state but can drill into immutable lineage.

8. Optional AI receives references to structured objects and returns explanation/hypothesis objects that cite evidence IDs.

## 12.3 Storage Strategy

Structured relational store for durable domain objects, indexes and references (Android Room/SQLite is suitable for MVP).

Chunked raw capture files for high-rate bus data with manifest/hashes rather than forcing every frame into relational rows.

Downsample tiers for long-term visualization; authoritative evidence referenced by preserved calculations must remain available or be compacted with integrity-preserving summary evidence.

Append-only audit/service objects; mutable “latest state” is a projection/materialized view, not the source of truth.

# 13. ESP32 Gateway and Signal Discovery

## 13.1 Gateway Hardware Requirements

ESP32-class controller with supported TWAI peripheral and a proper automotive CAN transceiver [S3].

Protected automotive power input suitable for vehicle electrical transients; fused installation and ignition/accessory-state strategy to avoid battery drain.

DLC3 harness that preserves normal diagnostic-port use and avoids loading the bus beyond specification.

Optional microSD/local flash capture ring buffer.

BLE baseline transport to Android; Wi-Fi/USB are optional higher-throughput transport paths.

Hardware revision ID and calibration/config identifiers exposed to Android.

## 13.2 Firmware Services

| Firmware module | Requirement |
| --- | --- |
| Bus manager | Initialize TWAI, receive frames, error counters, bus-off recovery, conservative filters. |
| Diagnostic requester | Only known allowlisted read requests; rate limits; no Active Tests / control commands. |
| Capture service | Raw frame/log capture, sequence numbers, timestamp, ring buffer and capture marker events. |
| Decoder assist | Optional lightweight decoding for immediate transport efficiency, while raw bytes remain authoritative. |
| Transport | Framed BLE/Wi-Fi/USB protocol with version negotiation, flow control, checksums and reconnect behavior. |
| Health service | Uptime, reset reason, queue depth, dropped frames, bus errors, storage state, voltage/power status if sensed. |
| OTA manager | Signed application/config update, inactive partition, self-test, validate or rollback [S4][S5]. |
| Safety policy | Read-only/passive mode default; transmit functions require explicit allowlist and experiment context. |

## 13.3 Protocol / Rate Detection Strategy

Although the initial target is expected to use ISO 15765-4 at DLC3 [S2], the discovery tooling should be robust enough to diagnose configuration problems instead of assuming a working bus. Detection must privilege passive observation and avoid noisy transmit sweeps on a live vehicle.

1. Boot into passive capture; record transceiver/bus state and error counters.

2. Attempt configured/known target profile first. Any fallback candidate bitrate/identifier mode must initially be listen-only.

3. Score candidate configuration using valid-frame rate, error rate, stable identifiers, plausible inter-arrival timing and repeatability.

4. Only after a valid passive lock may the diagnostic requester issue a minimal allowlisted read request.

5. Capture request/response statistics, errors and decoded plausibility into a Discovery Experiment.

6. Replay the bundle offline against candidate decoder/config changes before another live trial whenever possible.

## 13.4 Autonomous Iteration Boundary

Figure 3. Controlled decoder/protocol feedback loop.

Agent authority boundary The analysis agent may rank configurations, identify byte-level correlations, generate a decoder diff, suggest a new capture procedure or prepare a signed gateway firmware/config artifact. Activation on the live vehicle requires deterministic safety checks and an explicit approval policy. The agent never receives an unrestricted “send CAN frame” primitive.

## 13.5 Firmware OTA

Use an A/B OTA partition strategy compatible with ESP-IDF OTA facilities; new image boots into pending-verification state [S4].

Run post-boot self-tests: storage, transport, TWAI initialization, config parse, watchdog health, optional passive frame reception. Mark image valid only after checks pass; otherwise roll back [S4][S5].

Firmware image and configuration pack must be signed/hashed; Android verifies package metadata before transfer. Secure boot / signed-update options are hardware-revision dependent and must be configured intentionally [S5].

OTA event is stored in both Android audit log and gateway boot metadata. Firmware update must not imply or perform an update of Toyota ECUs.

# 14. Android Application Architecture

## 14.1 Recommended Module Boundaries

| Module | Responsibility |
| --- | --- |
| :app | Compose navigation, app lifecycle, dependency wiring. |
| :vehicle-domain | Vehicle/component/maintenance/finding domain models; no Android UI dependencies. |
| :gateway-protocol | BLE/Wi-Fi/USB framing, message schemas, connection state machine. |
| :signal-registry | Vehicle Signal Pack parser, decoder/scaling, quality rules. |
| :capture | Raw capture storage, replay reader, session/capture manifest. |
| :timeseries | Signal persistence/downsampling/query. |
| :maintenance | Schedule engine, component lifecycle, service/inspection logic. |
| :equations | Deterministic expression/rule runtime, version registry, calculation runs. |
| :diagnostics | DTCs, findings, experiment scoring, gateway health. |
| :ui-drive | Low-distraction driving surfaces. |
| :ui-garage | Maintenance, timeline, equation inspector, signal explorer, diagnostics. |
| :ai | Provider abstraction and evidence-bound prompts/contracts; optional. |
| :export | Backup/report package creation/import. |
| :test-replay | Golden captures, simulator sources and end-to-end deterministic replay tests. |

## 14.2 Background Connectivity

Android background execution and BLE behavior changes across OS versions. The implementation shall use platform-supported connection/background primitives, foreground services only when justified/visible, and WorkManager for deferrable persistent work. Current Android guidance explicitly describes BLE background constraints and WorkManager/foreground-service considerations [S6][S7].

## 14.3 State Model

GatewayConnection = Disconnected ¦ Connecting ¦ Connected ¦ Degraded ¦ Updating ¦ Error VehicleMotion = Unknown ¦ Parked ¦ Moving DataFreshness = Live ¦ Stale ¦ Missing MaintenanceState = Healthy ¦ Watch ¦ Inspect ¦ DueSoon ¦ Due ¦ Urgent ¦ Critical ¦ Unknown FindingState = Open ¦ Acknowledged ¦ Monitoring ¦ Resolved ¦ Dismissed ValidationStatus = Experimental ¦ BenchValidated ¦ VehicleValidated ¦ Calibrated

# 15. AI Analysis Layer

## 15.1 AI Responsibilities

Explain a deterministic finding in natural language while citing internal evidence IDs.

Compare present behavior to historical baseline and summarize the most relevant deltas.

Suggest diagnostic next steps and what additional measurement would increase confidence.

Analyze unknown/candidate CAN signals from labeled captures and propose decoder hypotheses.

Analyze gateway failure reports and propose a configuration/firmware diff for the next experiment.

Generate mechanic handoff summaries and maintenance planning summaries.

Answer questions about “why”, “when”, “what changed”, and “what evidence is missing”.

## 15.2 AI Prohibitions

AI may not be the sole generator of authoritative health scores or service intervals.

AI may not fabricate a sensor measurement, Toyota service specification, part number or threshold.

AI may not directly write arbitrary CAN frames or invoke hidden vehicle-control commands.

AI may not silently edit historical service records, calculation runs or equation definitions.

AI must label general automotive knowledge separately from facts measured on this vehicle.

AI output that could influence a critical stop/service decision must be downstream of deterministic validated logic, not upstream of it.

## 15.3 Evidence-Bound Response Contract

ai_claim: text: "Rear suspension compressor runtime has increased relative to baseline." claim_type: VEHICLE_OBSERVATION_INTERPRETATION evidence_refs: [metric:compressor.fill_time_30d, calc:calc_123] confidence: 0.91 hypothesis: false ai_claim: text: "A leak is one plausible cause." claim_type: HYPOTHESIS evidence_refs: [finding:air_suspension_degradation_42] confidence: 0.68 hypothesis: true

# 16. Alerts and Driver Safety

| Level | Behavior |
| --- | --- |
| Critical | Immediate potential risk based on validated deterministic criteria. Present full-screen/simple instruction; never rely solely on AI. |
| Urgent | Service/inspection should occur promptly; explain consequence and confidence. |
| Due | Manufacturer/configured maintenance interval reached. |
| Inspect | Physical inspection required; condition is unknown or evidence insufficient. |
| Watch | Developing trend; no immediate action required; show reason and trend. |
| Info | Service record, update, calibration or non-actionable system event. |

While moving, notifications are concise and tappable only into a safe summary; deep explanation waits until parked.

Alert hysteresis and deduplication prevent threshold chatter.

Gateway/Android failures must not masquerade as vehicle failures. “Data unavailable” is a separate state.

Where the factory cluster warning exists, the app may supplement it but must not claim to replace the factory warning system.

# 17. Security, Privacy and OTA

| Control | Requirement |
| --- | --- |
| Vehicle-bus isolation | No unrestricted transmit API. Diagnostic request allowlist, rate limits, moving-vehicle policy and experiment context enforced in gateway firmware. |
| Firmware integrity | Signed/hash-verified gateway firmware/config; A/B rollback; version compatibility checks [S4][S5]. |
| Local data | Vehicle health and service data stored locally by default; encryption at rest should use Android-supported secure storage/key mechanisms for sensitive keys/tokens. |
| AI privacy | AI disabled by default can remain a supported mode; when remote AI is enabled, user controls which structured evidence/raw captures may leave device. |
| Secrets | API keys/tokens never stored in source-controlled configuration packs or gateway plaintext logs. |
| Exports | Include checksums and manifest; optional redaction of VIN/location/raw trip data. |
| Audit | All firmware/config/equation activation and authoritative service-history mutations logged. |
| Recovery | Backup/restore and gateway rollback paths must be tested before field updates. |

# 18. Non-Functional Requirements

| ID | Category | Requirement |
| --- | --- | --- |
| NFR-001 | Availability | Android app must launch and show last known vehicle state even when gateway is disconnected; stale state visibly marked. |
| NFR-002 | Startup | Gateway and Android ingestion must recover automatically after normal power cycles without manual re-pairing under the same trusted-device configuration. |
| NFR-003 | Latency | Live dashboard target: routine live signal updates visible within 500 ms of receipt at Android; safety/critical UI path measured separately. |
| NFR-004 | Sampling integrity | Every source tracks expected cadence, observed cadence, dropped/late samples and stale timeout. |
| NFR-005 | Time integrity | Monotonic timestamps used for intervals/integration; wall-clock changes must not corrupt duration calculations. |
| NFR-006 | Data durability | Accepted maintenance, inspection, lifecycle, equation and audit objects are transactionally durable. |
| NFR-007 | Replay determinism | Same capture + same signal/equation/config versions produce the same derived output within declared numeric tolerance. |
| NFR-008 | Storage control | Raw high-rate capture retention is bounded; user-visible permanent history remains small enough for multi-year retention. |
| NFR-009 | Power | Gateway must have a defined sleep/power strategy and report power state; installation must avoid excessive parked battery drain. |
| NFR-010 | Thermal | Gateway records internal temperature when hardware supports it and survives expected in-cabin/vehicle installation conditions selected during hardware design. |
| NFR-011 | Resilience | Bus-off/transport disconnect/storage-full scenarios degrade gracefully and generate gateway-health events rather than corrupting vehicle health. |
| NFR-012 | Usability | Critical owner workflows — see due maintenance, explain a finding, add service, add inspection — require no developer tools. |
| NFR-013 | Accessibility | Text scales reasonably; meaning is not encoded by color alone; touch targets meet Android accessibility guidance where practical for the installed display. |
| NFR-014 | Observability | Gateway and Android expose structured logs, counters and version info sufficient to diagnose missing data without attaching a debugger. |
| NFR-015 | Compatibility | Signal/equation/config packs declare minimum/maximum compatible app and firmware versions. |
| NFR-016 | Testability | All calculation and maintenance logic can run against synthetic/replay input without physical vehicle. |

# 19. Test, Validation and Replay Strategy

## 19.1 Test Pyramid

| Layer | Required tests |
| --- | --- |
| Unit | Signal byte decoding/scaling; unit conversion; due calculations; equation runtime; confidence factors; lifecycle resets; alert hysteresis. |
| Property / fuzz | Malformed gateway frames, truncated capture bundles, invalid equation inputs, unit mismatches, overflow/NaN, timestamp disorder. |
| Replay | Golden captures from the target vehicle; expected decoded signals and findings pinned by version. |
| Integration | Android-to-simulated-gateway; disconnect/reconnect; backpressure; storage full; app reboot; firmware-version mismatch. |
| Bench CAN | ESP32 + transceiver against CAN simulator/test harness before live vehicle; validate bus-off and recovery behavior. |
| Parked vehicle | Passive enumeration/capture, allowlisted read requests, signal plausibility, no unintended bus traffic. |
| Controlled drive | Data freshness, trip segmentation, thermal/usage features, UI motion restrictions. No experimental actuation. |
| Service calibration | Enter physical measurements before/after service and verify lifecycle reset/model behavior. |
| OTA | Good image, corrupted image, interrupted transfer, failed self-test, rollback, incompatible config, low-power precondition. |
| AI | Evidence-citation checks, unsupported-claim detection, red-team prompts for arbitrary CAN control or history mutation. |

## 19.2 Golden Capture Program

Create labeled captures for ignition off/on, engine start, idle warmup, rev/load changes, short drive, steady cruise, braking, steering events, 4WD state changes (only normal driver controls), HVAC/A/C states, suspension settling/height behavior and tire-pressure changes when safely reproducible.

For each capture record external ground truth where practical: scan-tool value, thermometer, multimeter, pressure gauge, physical measurement or manual event marker.

Use captures to validate signal identity, scaling, sign/endian, update rate and plausible operating range.

Never promote an experimental decoder to “vehicle validated” only because the graph looks plausible; require independent corroboration.

## 19.3 Signal Discovery Scoring

| Metric | Example purpose |
| --- | --- |
| Frame validity / bus errors | Reject wrong bitrate/configuration. |
| Stable arbitration IDs | Identify recurring ECU traffic and source candidates. |
| Correlation to labeled event | Find fields that change with brake, speed, height, temperature, etc. |
| Monotonic/plausible scaling | Reject candidate transforms that violate physical expectations. |
| Cross-validation with scan tool | Corroborate candidate decoded value. |
| Replay stability | Same decoder produces same value on the same capture. |
| Coverage / freshness | Ensure the signal is available often enough for intended model. |

# 20. Delivery Plan / Engineering Epics

| Epic | Deliverable |
| --- | --- |
| E0 — Repository + contracts | Monorepo/module skeleton; domain IDs; protobuf/CBOR/JSON gateway envelope decision; schema versioning; CI; simulator source. |
| E1 — Android local truth store | Room schema; migrations; vehicle profile; component taxonomy; service/inspection/part ledger; audit events. |
| E2 — Gateway read-only capture | ESP32 TWAI receive, protected hardware interface, gateway health, capture manifest, BLE transport, simulator tests. |
| E3 — Ingest + replay | Android connection state machine, raw bundle persistence, sequence/dedup, replay engine, synthetic data. |
| E4 — Vehicle Signal Pack | Registry schema/runtime, generic OBD baseline, target-vehicle enumeration tools, Toyota-enhanced signal pack as discovered/validated. |
| E5 — Maintenance schedule engine | Versioned schedule configuration, due state, applicability, severe-use feature interface. |
| E6 — Equation + lineage engine | Immutable equation definitions, calculation runtime, evidence graph, confidence, historical replay/recalculation. |
| E7 — Owner UI | Home, maintenance queue, component detail, service/inspection wizard, history timeline. |
| E8 — Engineering UI | Signal Explorer, capture browser, Equation Inspector, gateway status, experiment/replay comparison. |
| E9 — First condition models | Tires, battery/crank, cooling/warmup, transmission thermal, air suspension, fuel trim/misfire; each behind validation flag. |
| E10 — OTA + configuration activation | Signed image/config metadata, Android transfer UI, A/B rollback/self-test, audit. |
| E11 — AI evidence assistant | Structured read API, evidence-linked explanations, capture-analysis proposal format, guardrails. |
| E12 — Export / handoff | Backup package, human-readable mechanic report, import/conflict handling. |
| E13 — Added sensor framework | Sensor provisioning/calibration/battery, A/C pressure, current/voltage, temp and IMU adapters. |

## 20.1 Recommended Development Order

1. Build the schema, simulator and replay harness before live-vehicle feature development.

2. Build ESP32 passive capture and Android ingest; prove lossless-ish capture accounting and reconnect behavior.

3. Create the Signal Registry and populate only signals independently validated on the target vehicle.

4. Implement service ledger + schedule engine so the app is useful before predictive models exist.

5. Implement Equation Registry + Calculation Runs + Evidence Inspector before adding many health models.

6. Add owner dashboard and maintenance queue on top of deterministic truth objects.

7. Add condition models one subsystem at a time, starting with high-observability items: tire pressure/leak, battery/crank, coolant/warm-up, transmission temperature if available, rear air suspension.

8. Add signed OTA and discovery experiment tooling only after baseline capture/replay is stable.

9. Add AI after evidence structures are stable, so AI consumes facts instead of defining them.

# 21. MVP Acceptance Criteria

| ID | MVP gate |
| --- | --- |
| AC-01 | App operates with simulated data and with a real gateway without changing UI/domain code. |
| AC-02 | ESP32 can capture/read target bus using the verified configuration and report zero/known-bounded dropped frames under the validated test load. |
| AC-03 | Android persists raw capture, decoded samples and gateway version metadata, then replays the capture offline. |
| AC-04 | At least the first verified set of maintenance-relevant signals appears in Signal Explorer with source, units, freshness and quality. |
| AC-05 | Vehicle profile supports the target 2005 4Runner and prevents engine-specific maintenance rules from running when engine identity is unknown. |
| AC-06 | Configured Toyota maintenance items appear in queue with mileage/time basis and can be completed via service records. |
| AC-07 | Parts/fluids/inspections create permanent component lifecycle history. |
| AC-08 | At least five derived metrics use the Equation Registry and Calculation Run model; each can be reproduced from persisted evidence. |
| AC-09 | Home → component → explain → evidence path works for each derived metric in scope. |
| AC-10 | Historical calculation remains unchanged after a newer equation version is activated; optional recalculation creates a new linked run. |
| AC-11 | Data loss/reconnect/storage/stale-signal conditions are visibly distinguished from a vehicle fault. |
| AC-12 | No arbitrary CAN transmit primitive exists in the Android app; gateway diagnostic requests are allowlisted and logged. |
| AC-13 | Gateway OTA supports validated-image success and intentional self-test failure rollback on bench hardware. |
| AC-14 | Export produces a self-contained service/health package that includes version manifest and checksums. |
| AC-15 | AI can be completely disabled without loss of core function; when enabled, vehicle-specific claims expose evidence references. |

# 22. Risks and Open Decisions

| Risk | Failure mode | Mitigation |
| --- | --- | --- |
| R1 — Toyota enhanced PID availability | Not every desired maintenance signal is available through generic OBD or the same diagnostic path. | Enumerate target ECUs and build signal pack from verified captures; mark unavailable signals and use inspection/added sensors. |
| R2 — CAN/diagnostic bus safety | Poorly designed probing can create bus errors or interfere with vehicle systems. | Passive-first discovery, hardware transceiver/protection, allowlisted requests, rate limiting, no Active Tests. |
| R3 — False predictive confidence | A clean-looking equation can imply more certainty than evidence supports. | Separate measured/estimated/scheduled/inspection states; confidence; validation levels; “unknown” supported. |
| R4 — Android background restrictions | BLE/background behavior changes across OS versions. | Follow current Android platform guidance; keep connection strategy modular; installed head unit can use device-specific policies where appropriate [S6][S7]. |
| R5 — Storage growth | Raw CAN capture can become large quickly. | Ring buffers, capture-on-demand, tiered retention, downsampling and evidence-aware compaction. |
| R6 — OTA failure | Gateway update could brick collection or leave incompatible config. | A/B OTA, signed images, compatibility metadata, rollback and bench validation [S4][S5]. |
| R7 — Added sensor calibration | Cheap sensors can drift and create false maintenance findings. | Per-sensor calibration state, quality grade, cross-check with reference tool, replaceable sensor lifecycle. |
| R8 — Vehicle configuration ambiguity | Unknown engine/trim/options cause wrong rules. | Onboarding resolution + “unknown” states; never activate incompatible schedule/equation silently. |
| R9 — Model reset semantics | Replacing one component may invalidate only part of a learned baseline. | Each equation declares reset dependencies; service events generate targeted baseline boundaries. |
| R10 — AI overreach | Agent may sound certain about weak evidence or propose unsafe experimentation. | Evidence contract, hypothesis labels, no raw vehicle-write tool, proposal diff + safety gate. |

## 22.1 Open Decisions Before Hardware Freeze

Exact ESP32 board/module and CAN transceiver/protection/power design for permanent vehicle installation.

Primary production Android-to-gateway transport: BLE only vs BLE + Wi-Fi for bulk capture transfer.

MicroSD on gateway vs short flash ring buffer + immediate Android streaming.

How vehicle movement/parked state is authoritatively determined for UI/OTA gating.

Whether the target Android device is always powered with ignition or has independent standby power.

Whether cloud backup is desired in V1 or kept post-MVP.

Exact AI provider/local model deployment after evidence API is stable.

Which added sensors are installed first after native-signal enumeration.

# 23. Initial Backlog

| ID | Backlog item | Epic | Pri |
| --- | --- | --- | --- |
| B-001 | Create repo and shared versioning conventions for app, gateway, signal pack and equations. | E0 | P0 |
| B-002 | Define gateway transport envelope and sequence/CRC/error semantics. | E0/E2 | P0 |
| B-003 | Create Android simulator gateway source that emits realistic timestamped samples. | E0/E3 | P0 |
| B-004 | Implement Vehicle, Component, InstalledPart, MaintenanceEvent, Inspection and AuditEvent Room entities/migrations. | E1 | P0 |
| B-005 | Implement service-entry and inspection-entry UI with amendment workflow. | E1/E7 | P0 |
| B-006 | Bring up ESP32 TWAI in passive receive with bus health counters on bench. | E2 | P0 |
| B-007 | Validate target vehicle DLC3/CAN configuration and save first signed capture manifest. | E2/E4 | P0 |
| B-008 | Implement BLE gateway discovery/pair/connect/reconnect and version handshake. | E3 | P0 |
| B-009 | Persist raw capture chunks + manifest and replay them into ingest pipeline. | E3 | P0 |
| B-010 | Define SignalDefinition schema and first generic OBD signal pack. | E4 | P0 |
| B-011 | Build signal-validation worksheet/tool: scan-tool ground truth vs candidate decoder. | E4 | P0 |
| B-012 | Enumerate target ECUs and candidate Toyota Data List signals; version results. | E4 | P0 |
| B-013 | Model Toyota maintenance schedule as configuration with applicability. | E5 | P0 |
| B-014 | Implement maintenance due calculation and queue. | E5/E7 | P0 |
| B-015 | Implement EquationDefinition + CalculationRun persistence and deterministic runtime. | E6 | P0 |
| B-016 | Implement Evidence Trace API resolving a derived metric to raw observations. | E6 | P0 |
| B-017 | Build Home and Component Detail views on synthetic/replay data. | E7 | P0 |
| B-018 | Build History Timeline with service/inspection/finding/calculation version markers. | E7 | P0 |
| B-019 | Build Signal Explorer and Equation Inspector. | E8 | P0 |
| B-020 | Implement tire pressure temperature normalization + leak-rate experimental model. | E9 | P1 |
| B-021 | Implement crank-event detector + battery trend experimental model. | E9 | P1 |
| B-022 | Implement coolant warm-up baseline model. | E9 | P1 |
| B-023 | Verify ATF temperature source and implement thermal exposure if available. | E9 | P1 |
| B-024 | Enumerate air-suspension signals and implement compressor/height baseline model. | E9 | P1 |
| B-025 | Implement capture experiment object, comparison metrics and replay score. | E8/E9 | P1 |
| B-026 | Implement signed ESP32 OTA package metadata, transfer and rollback bench tests. | E10 | P1 |
| B-027 | Define AI read-only evidence API and claim/evidence response schema. | E11 | P1 |
| B-028 | Implement AI capture-report analyzer that outputs reviewable config/decoder proposal diff. | E11 | P1 |
| B-029 | Implement export package + human-readable mechanic summary. | E12 | P1 |
| B-030 | Select/calibrate first added sensors (A/C pressure and electrical/current are high-value candidates). | E13 | P2 |

## 23.1 Definition of Done for Any New Signal

Registry entry exists with stable ID, source, decoder/scaling, unit and applicability.

At least one recorded target-vehicle capture contains the signal.

Decoded value is corroborated against an independent reference or labeled physical behavior.

Plausible range and freshness expectations are defined.

Quality/failure behavior is tested with missing/malformed/stale data.

If used in an equation, equation applicability and reset dependencies are defined.

Golden replay test pins expected decoding for the validated registry version.

## 23.2 Definition of Done for Any New Health Model

Deterministic Equation Definition is versioned and human-readable.

Inputs are stable signal/feature IDs with unit and quality constraints.

Output meaning and truth boundary are explicit: measured, estimated, schedule or hypothesis.

Confidence model is defined and tested under missing/stale/noisy evidence.

Calculation Run persists exact inputs, intermediates, output and version metadata.

UI exposes Explain/Evidence path.

Model is replay-tested on both positive/negative or normal/abnormal examples where available.

Validation status remains Experimental until target-vehicle evidence justifies promotion.

# 24. Sources and Technical References

These references support platform/interface constraints used in the PRD. Vehicle-specific PIDs, service constants and thresholds still require verification against the target 2005 4Runner and applicable Toyota service/maintenance information before production use.

[S1] Toyota Owners — Manuals and Warranties — https://www.toyota.com/owners/warranty-owners-manuals/

[S2] 2005 Toyota 4Runner repair information mirror — DLC3 / ISO 15765-4 CAN — https://charm.li/Toyota/2005/4%20Runner%204WD%20V6-4.0L%20%281GR-FE%29/Repair%20and%20Diagnosis/Powertrain%20Management/Computers%20and%20Control%20Systems/Testing%20and%20Inspection/Scan%20Tool%20Testing%20and%20Procedures/DLC3%20%28Data%20Link%20Connector%203%29/

[S3] Espressif ESP-IDF — Two-Wire Automotive Interface (TWAI), ESP32 — https://docs.espressif.com/projects/esp-idf/en/stable/esp32/api-reference/peripherals/twai.html

[S4] Espressif ESP-IDF — Over The Air Updates (OTA), ESP32 — https://docs.espressif.com/projects/esp-idf/en/stable/esp32/api-reference/system/ota.html

[S5] Espressif ESP-IDF — ESP32 Security Overview / secure update mechanisms — https://docs.espressif.com/projects/esp-idf/en/stable/esp32/security/security.html

[S6] Android Developers — Communicate in the background with Bluetooth LE — https://developer.android.com/develop/connectivity/bluetooth/ble/background

[S7] Android Developers — Background tasks / persistent work / WorkManager — https://developer.android.com/develop/background-work/background-tasks

[S8] Verma et al. — CAN-D: A Modular Four-Step Pipeline for Comprehensively Decoding CAN Data — https://arxiv.org/abs/2006.05993

# Appendix A — Initial Screen Inventory

| ID | Screen |
| --- | --- |
| A01 | Boot / connection status |
| A02 | Home / health summary |
| A03 | Maintenance queue |
| A04 | Maintenance item detail |
| A05 | System/component tree |
| A06 | Component detail |
| A07 | Installed part / fluid lifecycle |
| A08 | Service entry wizard |
| A09 | Inspection entry wizard |
| A10 | Unified history timeline |
| A11 | Finding detail |
| A12 | DTC history |
| A13 | Signal Explorer |
| A14 | Signal detail / graph |
| A15 | Equation Inspector |
| A16 | Evidence Trace |
| A17 | Calculation Run detail |
| A18 | Trip/session detail |
| A19 | Gateway health |
| A20 | Capture browser |
| A21 | Capture detail / replay |
| A22 | Discovery experiment |
| A23 | Experiment comparison |
| A24 | Firmware / OTA |
| A25 | Sensor provisioning/calibration |
| A26 | Vehicle profile / applicability |
| A27 | Data retention / backup/export |
| A28 | AI assistant with evidence drawer |
| A29 | Developer settings |
| A30 | About/version manifest |

# Appendix B — Example “Why?” Drill-Down

1. Home shows “Rear Air Suspension — Watch — confidence 91%.”

2. Component Detail shows “Compressor fill time +32% vs lifecycle baseline” and “parked height decay 1.4 mm/h.”

3. Explain shows finding rule AIR_SUSP_DEGRADATION@1.2 and the major contributors.

4. Equation Inspector shows the exact expression/constants and Calculation Run ID.

5. Input detail shows the 30-day feature calculation, sample count, exclusions and missing-data ratio.

6. Evidence Trace resolves each feature to signal samples and raw observations/capture IDs.

7. History overlays prior service: rear air spring replaced → baseline reset boundary.

8. After repair, new lifecycle starts; prior finding remains historically visible and linked to the repair event.

# Appendix C — Product Invariants

No service/inspection history is silently overwritten.

No derived metric without a calculation/evidence path.

No physical measurement presented as an estimate or vice versa.

No “last value” presented as live after its stale timeout.

No AI claim about this vehicle without evidence or a visible hypothesis label.

No arbitrary CAN transmit interface in production Android UI.

No gateway OTA without integrity verification and recovery path.

No engine/trim-specific maintenance rule when applicability is unresolved.

No model promotion from Experimental without validation evidence.

No maintenance rule encoded only in UI code; it must be versioned configuration/equation data.

Development starting point Start with E0–E4 in parallel only where interfaces are already defined: schema/simulator/replay first, passive ESP32 capture second, then target-vehicle signal enumeration. Build maintenance history and equation lineage before predictive scoring so every later model inherits the auditability contract.